# Chapter 4: Implementation and Results

# 4.1 Implementation Details and Model Architecture

In this chapter, we present the technical implementation and results of the speaker diarization system. Our approach utilizes a hybrid Convolutional Neural Network (CNN) and Long Short-Term Memory (LSTM) model, targeting both spectral and temporal characteristics of human speech. The goal is to effectively segment and identify speakers in an audio stream.
We utilized a 174.59-second audio file sampled at 16kHz (mono), which was processed into Mel-Frequency Cepstral Coefficients (MFCCs) with 13 coefficients per frame. The frame length was 512 samples with a hop length of 256, yielding approximately 10,913 frames.
The model architecture consists of the following layers:
- A Conv1D layer with 64 filters and kernel size of 3
- Batch normalization to stabilize training
- MaxPooling1D with a pool size of 2
- An LSTM layer with 128 units to capture temporal dependencies
- Dropout layer (0.3) to reduce overfitting
- Dense layers with softmax activation for classification
Training was conducted on CPU (CUDA disabled) for broader compatibility. We generated simulated speaker labels 
that alternated every 50 frames (i.e., speaker 0 and 1 switching). The model was trained using an 80/20 train-test split with 20-frame sequences, for 10 epochs and a batch size of 16. Adam optimizer and categorical cross-entropy 
loss function were employed.

To support diverse hardware capabilities, we developed two interfaces: a Streamlit web application and a Tkinter desktop application. The Streamlit app caters to users with high-performance systems, offering interactivity and 
visualization, while the Tkinter version ensures offline, low-resource usage suitable for less powerful devices.

Training and validation accuracy over 10 epochs:

| Epoch | Training Accuracy (%) | Validation Accuracy (%) |
| --- | --- | --- |
| 1 | 56.89 | 61.36 |
| 2 | 70.62 | 71.82 |
| 3 | 80.23 | 75.40 |
| 4 | 85.97 | 77.65 |
| 5 | 89.83 | 87.10 |
| 6 | 92.00 | 93.30 |
| 7 | 93.53 | 92.98 |
| 8 | 94.22 | 94.91 |
| 9 | 94.88 | 91.14 |
| 10 | 95.18 | 88.48 |

The model showed consistent improvements through epochs, although a dip in validation accuracy in later epochs 
suggests minor overfitting. Overall, the model demonstrated strong speaker segmentation capabilities.

In the next sections, we delve into clustering quality via silhouette score analysis, performance comparisons, 
error assessments, and visualization results across the Streamlit and Tkinter applications.